REMOVAL OF FIXED AND MOVABLE ASSETS

Request for Proposals and/or Project Specifications

Use in Request for Proposals or add to relevant specification sections.  These are frequently:

SECTION 01732 - SELECTIVE DEMOLITION or

SECTION 02070 - DEMOLITION

Add to Definitions Paragraph in Part 1 of the Specifications:

Fixed and Movable Assets:  Items that can be removed intact during pre-demolition or demolition with the potential to be re-used.  Examples of such items include, but are not limited to, furniture and furnishings, windows and doors, casework, cabinetry, bathroom fixtures, and kitchen appliances and other items specified in this Section.

Add to Submittals Paragraph in Part 1 of the Specifications:

Market or Outlet for Reuse of Fixed and Movable Assets:  Submit qualifications of organization designated to facilitate reuse of fixed and movable assets for Owner’s approval.  Include a list of recent projects where items have been reused and markets or recipients where fixed and movable assets have been delivered.  Include name of person assigned to this Project.  Acceptable organizations include, but are not limited to, the following:  Institution Recycling Network, 7 South State Street, Concord, NH  03301, 866-229-1962, 603-229-1960 fax, www.irnsurplus.com.

List of Fixed and Movable Assets to Be Reused and/or Recycled:  Inventory site and provide preliminary list of fixed and movable assets that can be reused and/or recycled, including quantities and any costs or savings to the project budget.  After consultation with Owner, provide final list of fixed and movable assets to be reused and/or recycled, specifying which are to be handled for reuse and which are to be handled for recycling.  Provide tracking and reporting forms as specified under Paragraph XXX:  Removal of Fixed and Movable Assets.

Add Paragraph in Part 2 of the Specifications:

2.1	REMOVAL OF FIXED AND MOVABLE ASSETS

- General:  All fixed and movable assets specified below shall be considered for reuse and/or recycling and shall be included in the preliminary list of fixed and movable assets that can be reuse and/or recycled.  After review by Owner, all materials on the final list of fixed and movable assets to be reused and/or recycled shall be removed.  Those designated for reuse shall be removed intact and shall be distributed by sale or donation to parties who are capable of assuring their reuse.  Those designated for recycling shall be removed and handled using methods and equipment appropriate to each material and the markets to which they are being provided.

- Materials:  Fixed and movable assets to be removed for reuse and/or recycling shall include:

(Add to, modify, or delete from this list as appropriate to specific project)

  - Furniture (desks, tables, chairs, file cabinets, bookshelves, etc.);

  - Beds and mattresses;

  - Bed linens, pillows, towels, shower curtains, and other residential soft goods;

  - Display cabinets, blackboards, whiteboards, etc.;

  - Library furnishings and equipment, including but not limited to shelving, carrels, tables, workstations, etc.;

  - Laboratory benches and cabinets; laboratory shelving;

  - Partition/panel/cubicle systems;

  - Windows (sash only);

  - Doors (including or excluding frames);

  - Wood millwork, paneling, wainscoting, and other similar interior finishes;

  - Wood, marble and other flooring;

  - Bathroom plumbing fixtures (toilets and sinks);

  - Bathroom stall partitions;

  - Mirrors;

  - Cabinets and countertops (kitchen, bath, storage, etc.);

  - Sinks (kitchen, bath, other; in-counter and freestanding);

  - Dining and/or cafeteria tables and chairs, foodservice lines, and other dining and cafeteria furniture and equipment;

  - Food preparation and foodservice tables;

  - China-, glass-, and silverware; Serving dishes; Pots and pans; Cooking utensils;

  - Table cloths, dining linens, and similar soft goods;

  - Kitchen appliances (ovens, ranges, dishwashers, free-standing and walk-in refrigerators, chillers, etc.).

- Procedures:  Remove specified fixed and movable assets and manage their distribution, by sale or donation, to organizations that can assure that they will be reused or, if unsuitable for reuse, will be recycled.

  - Provide tools and manpower qualified to remove materials specified for reuse intact and in a condition that they may be reused.

  - Remove materials specified for reuse intact and in condition suitable for reuse; set aside in a location and in such a manner that they will not be damaged by other site operations.

  - Identify for-profit or not-for-profit organizations that can reuse or facilitate the reuse of materials.  Manage the distribution of fixed and movable assets to these organizations.

  - Identify markets to recycle fixed and movable assets that are unsuitable for reuse.  Manage removal and transportation of recyclable materials to these markets.

  - Track and report in a form acceptable to Owner the disposition of all fixed and movable assets, including organizations to which assets are distributed for reuse or recycling.  Tracking and reporting may be by piece count or by weight, as appropriate to the materials and their distribution.